IoT Device Profiles

| Name | Type | Feature of Interest | Property | Characteristics | Components | Justification |
| --- | --- | --- | --- | --- | --- | --- |
| The name of each IoT device | E.g., sensor, actuator, or device | What “thing” is being observed and/or acted upon? | Which aspect(s) of the “thing” is/are being observed and/or acted upon? | What are the characteristics of the device? | What are the components embedded in the device? | Why is the device selected for the proposed IoT solution? |
| LR-6XL Reader (an example of RFID reader) https://tagmaster.com/rfid-long-range-reader/lr-6xl-reader/ | device | vehicle | Vehicle access and identification | - Long-range reading (read range up to 14 meters) - Support an extensive variety of industry-standard protocols, including OSDP, Ethernet (TCP/IP), RS232, and RS485 serial communication. - Can be configured and controlled via the Ethernet interface, either locally or remotely via an Internet connection. - Robust and weatherproof design for outdoor use - Frequency hopping - Easy integration with other systems - Low power consumption - Easy to install and easy to use - Cost-effective | - Integrated antenna - Built-in controller - Built-in high intensity red and green LEDs - Built-in buzzer indicators | This device is used for vehicle access and identification, as it reads vehicle information stored on RFID tags. Its long-range detection capability, weatherproof design, and Ethernet-enabled configuration make it highly suitable for accurate and continuous operation in outdoor port environments. |
| Windshield Tag UHF RAIN (EPC GEN 2) (an example of RFID tag) https://tagmaster.com/rfid-long-range-reader/windshield-tag/ | device (tag) | vehicle | Vehicle information | - Battery-free operation - Has a read range up to 9 meters - Unique tag - Tamper-evident - Cost-effective | - RFID chip (Impinj Monza 4D) - Embedded antenna | This tag is used to store and transmit vehicle information for identification during port entry and weighing activities. Its battery-free, tamper-evident, and cost-efficient design allows secure, maintenance-free, and large-scale deployment across multiple trucks. |
| MarkTag Classic 2,45 GHz TAG (better option for tag) https://tagmaster.com/rfid-long-range-reader/marktag-classic/ | device (tag) | vehicle | Vehicle information | - using the 2.45GHz frequency band - Up to 10 metres - Can be read in any orientation | - Built-in battery - RFID chip | This tag is used to store and transmit vehicle information. Its orientation-independent reading capability, long-range performance, and robust outdoor stability ensure reliable communication even when vehicles are in motion or misaligned. |
| L6A Enterprise LPR Camera (an example of a recognition camera) https://www.avigilon.com/security-cameras/l6a | device | vehicle | Vehicle’s movement | - Able to capture plates all day - Low light capture range is up to 250 ft (76.2m). - Daylight capture range (20330 ft). - Greater flexibility, scalability, and ease-of-use | - Two image sensors (Context and LPR) - IR Illuminator | This device is used to monitor the movement of vehicles by capturing their license plates at different checkpoints. Its long-range, low-light, and weather-resistant characteristics enable reliable vehicle recognition and tracking throughout day and night in outdoor port environments. |
| H5A Rugged PTZ Camera (an example of a security camera) https://www.avigilon.com/security-cameras/h5a-rugged-ptz | device | vehicle port | Vehicle’s movement in the port Operational activities within the port | - 360° Surveillance & Zoom - Water, corrosion & dust protection - High wind resistance - Wide temperature range - Flexible installation - Advanced Video Analytics with Auto-Tracking | - CMOS sensor - IR Illuminator - Embedded PTZ (Pan-Tilt-Zoom) mechanism | This camera is used to monitor vehicle movement and overall port security. Its 360° coverage, auto-tracking analytics, and rugged outdoor design allow continuous surveillance of wide port areas under various environmental conditions. |
| OVS-50TNR – Vehicle detection sensor (an example of infrared motion sensors) https://www.optex.co.jp/e/products/vehicle-detection/photo-beam/ovs-50tnr.html https://www.bhphotovideo.com/c/product/1836262-REG/optex_ovs_50tnr_hybrid_photoelectric_50_dual.html | sensor | vehicle | Vehicle’s location on weighbridge | - Maintains stable performance in diverse weather conditions, including rain, snow, and fog. - Easy installation - Long battery life - Up to 15 meters (50 feet) point-to-point - No wiring required for operation on the transmitter side | - Transmitter unit - Receiver unit - Indicator LEDs | This sensor is used to detect the vehicle’s position on the weighbridge. Its infrared-based, weather-resistant, and long-range detection capability ensures accurate vehicle alignment before weight measurement, even under rain or fog. |
| CPD-M Digital load cell (an example of a weighing device) https://nwiweighing.com.au/products/cpd-m-digital-weighbridge-loadcell/ | sensor | vehicle | Vehicle weight | - Self-Diagnostics & Fault Detection - Self-stabilising system with anti-rotation device - Water and dust resistance - Stainless steel for corrosion protection | - 8 strain gauges - Disk (interrupts the flow of electricity between the bridge’s upper structure and the base) - Rodent-resistant cable - Filters (prevent interference from high frequency devices) | This device is used to measure the vehicle’s weight on the weighbridge. Its self-diagnostic capability, water- and dust-resistant stainless-steel housing, and anti-rotation stabilising system ensure accurate and reliable weight measurements under outdoor port conditions. |
| DGT1S Digital weight transmitter (an example of a data transmitter) https://nwiweighing.com.au/products/dini-argeo-dgt1s-multifunction-weight-transmitter/ | device | vehicle | Vehicle weight | - Waterproof design - Allow the direct reading and diagnostics of the connected weighing system - Easy integration with any automation system | - LED display - LED lights - Keyboard - 6 wires with sense | This device is used to transmit and display the measured vehicle weight from the load cell. Its waterproof design, real-time diagnostic capability, and easy integration with automation systems enable stable and accurate weight data communication between the weighbridge and the control platform. |
| Crodeon - All in one weather sensor (an example of an environmental sensor) https://www.crodeon.com/products/all-in-one-weather-sensor | sensor (integrated) | port | Temperature Humidity rainfall | - Measure multiple parameters at the same time - All data connected via a single cable (simplify integration and reduce the need for multiple connectors) | - A rain sensor - A wind sensor (a wind vane and an anemometer) - A temperature and humidity sensor | This sensor is used to observe environmental conditions that may influence loading and transportation activities in the port. Its ability to measure multiple parameters simultaneously and its single-cable integration make it ideal for providing real-time weather context with minimal installation complexity. |
| Dart3 Global BLE (an example of GPS device) https://www.digitalmatter.com/devices/dart3-global/ | sensor / GPS device | vehicle | Vehicle location | - Integrate with any third-party Bluetooth accessory for customizable sensor and condition monitoring - Features an internal backup battery in case of loss of power or tampering | - Cellular Antenna - GPS Antenna - 3-Axis Accelerometer (to detect movement, acceleration, high G-force events, and more) - Diagnostic LED - Bluetooth 5.2 gateway | This device is used to track the vehicle’s location and movement throughout the port and transportation routes. Its built-in GPS antenna, accelerometer, and Bluetooth gateway ensure precise tracking and reliable data transmission even during motion or power interruptions. |