Specification of Selected IoT Network Technologies and Computing Platforms

| IoT devices and network devices (e.g., router, hub, gateway, etc.) | Main considerations | Specific questions | Responses |
| --- | --- | --- | --- |
| LR-6XL Reader & MarkTag Classic 2, 45GHz TAG (an example of RFID tag and reader) | Determine the connectivity between selected IoT devices, gateways, routers, etc. | Why do selected devices need to be connected with each other and to the network? | The LR-6XL Reader communicates with the MarkTag Classic tag through wireless RF signals. It supports a read range of up to 14 meters. This communication does not require network technologies. |

| IoT devices and network devices (e.g., router, hub, gateway, etc.) | Main considerations | Specific questions | Responses |
| --- | --- | --- | --- |
| LR-6XL Reader & Edge device (e.g. a computer) | Determine the connectivity between selected IoT devices, gateways, routers, etc. | Why do selected devices need to be connected with each other and to the network? | The RFID reader at the gate must connect to the server to transmit detected tag information for access control processing. |
|  | Determine communication and network technologies required to support connectivity | What are the range and coverage requirements? | The network needs to cover the gate area and the nearby control room, where the Edge device is located, to ensure continuous and reliable transmission of tag information. Therefore, the chosen network technology should support short-range, stable connectivity. |
|  |  | What are the power consumption constraints? | The reader is a low-power IoT device. The Edge device (a computer) has a reliable power supply. |
|  |  | What is the required data rate (how much data needs to be transmitted over time)? | The system requires a low data rate due to the small size of RFID tag data (8-digit ID). |
|  |  | What are the latency requirements (how fast does data need to be transmitted)? | For near real-time access control, low latency is critical to ensure RFID reads trigger immediate responses, such as gate activation or security alerts. |
|  |  | What are the security and reliability requirements? | RFID systems handle sensitive access data, such as truck licence plate numbers, driver IDs, and names, requiring strong encryption and authentication to prevent unauthorised access. Also, network reliability is crucial, especially in a port environment where harsh weather conditions, such as strong winds, heavy rain, and high humidity, can impact connectivity. |
|  |  | What are the cost considerations? | Ethernet has higher upfront installation costs, but low ongoing and operational costs, making it cost-effective for stable, long-term deployments. |
|  |  | Which communication/network technologies are selected? | Ethernet is chosen for its stability, efficiency, and long-term cost-effectiveness in port gate operations. As a wired communication solution, it provides reliable short-range coverage, ensuring uninterrupted connectivity across the gate area. |
|  | Determine the connectivity between IoT devices and network devices in the edge/fog layer | Is an edge/fog layer required? Why? | Yes, an edge layer is required, because - low latency response is critical as the system must trigger instant gate operations and prevent access delays; - local processing ensures reliability as connecting to cloud may encounter network failures, especially in harsh weather conditions. |
|  |  | Which network device(s) should handle computing across the edge/fog layer? | The edge device (e.g., industrial computer) should handle edge computing: - The computer is located in the nearby control room, allowing real-time RFID authentication. |
|  |  | What data is transmitted between the IoT devices and network device(s)? | The vehicle-related information (e.g., licence plate number, driver ID, driver name) stored in the RFID tag is transmitted from the RFID reader to the edge device to identify the vehicle and authenticate its access to the port. |
|  |  | What data processing and/or analysis tasks should be performed in the edge/fog layer? | To authenticate the vehicle’s access to the port, the following data processing and analysis tasks are required: - Match RFID tag information with the local database to determine gate access (grant or deny). - Trigger real-time alerts if an unregistered tag is detected. |
|  |  | If the fog connects with a user application, what is the purpose of the connection? | Gate operation dashboard Purpose: To provide real-time monitoring of gate operations, including vehicle entry time, license plate number, and authentication status. |
|  |  | If the fog connects with a system, what is the purpose of the connection? | Access control system Purpose: To enable automated access control decisions and security monitoring by generating approval/denial responses based on authentication results, and triggering alerts for unauthorised vehicles. |
|  | Determine the connectivity to the cloud | Is a cloud layer required? Why? | Yes, a cloud layer is required to enable long-term data storage and perform historical and predictive analysis on truck arrivals. This helps with decisions such as scheduling truck arrivals and optimising port resource and capacity planning, which are not time-sensitive and can be processed outside the local environment. |
|  |  | What data should be transmitted to the cloud, and is it from IoT devices or edge/fog layer devices? | Only selected and aggregated data should be transmitted from the edge layer (industrial computer) to the cloud, including: - Truck Entry Logs: for analysing arrival pattern and optimising scheduling. - Unauthorised Access Attempts: for detecting suspicious behaviour patterns over time. |
|  |  | What data processing and/or analysis tasks should be performed in the cloud? | Arrival Pattern Analysis - Aggregate and analyse truck entry data over time to identify peak periods and congestion trends. Vehicle Access Behaviour Analysis - Analyse trends and patterns of unauthorised access attempts to assess risks and improve gate security protocols. |
|  |  | If the cloud connects with a user application, what is the purpose of the connection? | Vehicle security profiling application Purpose: To allow security staff to access historical vehicle entry data and identify unusual or suspicious patterns. |
|  |  | If the cloud connects with a system, what is the purpose of the connection? | Port access dashboard Purpose: To support port administrators in monitoring overall access activity and detecting access-related anomalies or trends. |

| IoT devices and network devices (e.g., router, hub, gateway, etc.) | Main considerations |  | Specific questions |  | Responses |  |
| --- | --- | --- | --- | --- | --- | --- |
| OVS-50TNR – Vehicle detection sensor (an example of an infra-red device) CPD-M Digital load cell (an example of a weigh-bridge weighing sensor) DGT1S Digital weight transmitter (an example of weight transmitter) Edge device IoT gateway (an example of a fog device) | Determine the connectivity between selected IoT devices, gateways, routers, etc. |  | Why do selected devices need to be connected with each other and to the network? |  | The load cell and digital weight transmitter connect to the Edge device to measure and transmit the truck's weight in real time. The vehicle detection sensor also connects to the Edge device to verify that the truck is correctly positioned on the weighbridge, ensuring accurate weight measurements. The Edge device aggregates this data and transmits it to the IoT gateway, where real-time analysis detects any weight anomalies. |  |
|  | Determine communication and network technologies required to support connectivity |  | What are the range and coverage requirements? |  | The load cell, weight transmitter and vehicle detection sensor are installed on the weighbridge, within a few meters from the Edge device in the nearby control room. The connection between the weighbridge station and the IoT gateway depends on their physical distance. In the port, where multiple weighbridge stations operate, the distance from each weighbridge to the central control office (housing the IoT gateways) ranges from 500 meters to 2 km. To ensure continuous and reliable data transmission, the network must provide full coverage across the port. Therefore, the chosen technology should support stable, medium range connectivity where needed. |  |
|  |  |  | What are the power consumption constraints? |  | The load cell and weight transmitter operate on low power. The vehicle detection sensor is self-powered, featuring a built-in battery designed for low power consumption. The Edge device in the nearby control room has a reliable power supply. The IoT gateway (industrial computer) in the port’s central control office, requires continuous power to process aggregated weighbridge data and detect anomalies in real time. |  |
|  |  |  | What is the required data rate (how much data needs to be transmitted over time)? |  | Weight data consists of small numerical values, requiring low bandwidth. Transmission does not require high data rates, as weight readings are sent periodically rather than continuously. The vehicle detection sensor also transmits small packets indicating whether the truck is correctly positioned. Since these readings are event-triggered rather than continuous, low bandwidth is sufficient. However, since multiple weighbridges send data to the IoT gateway, the network should handle aggregated sensor data without congestion. |  |
|  |  |  | What are the latency requirements (how fast does data need to be transmitted)? |  | When connecting the load cell, weight transmitter, and vehicle detection sensor to the edge device, low latency is essential for accurate real-time weight measurement processing. When connecting the edge device to the IoT gateway, low latency is also crucial for analysing and detecting abnormal weight cases in a near real-time setting. |  |
|  |  |  | What are the security and reliability requirements? |  | Data encryption is required to protect truck positioning and weight data during transmission. |  |
|  |  |  | What are the cost considerations? |  | <optional> |  |
|  |  |  | Which communication/network technologies are selected? |  | Ethernet is chosen to connect the load cell, weight transmitter, and vehicle detection sensor to the Edge device due to its low latency, ensuring real-time weight measurement processing with instant data transfer. It offers high reliability, free from interference, and provides stable, uninterrupted communication compared to wireless alternatives. As a wired connection, Ethernet enhances security by reducing tampering risks. With sufficient bandwidth, Ethernet can efficiently transmit small weight data packets. LTE-M is selected to connect the Edge device to the IoT gateway for its fast data transmission and low latency, enabling real-time fraud detection and anomaly analysis. It is scalable for long-distance communication, ensuring reliable connectivity across the entire port. |  |
|  | Determine the connectivity between IoT devices and network devices in the edge/fog layer |  | Is an edge/fog layer required? Why? |  | Yes, both edge and fog layers are required. The edge layer handles real-time validation of vehicle weight measurements, while the fog layer stores and manages recent local data at the IoT gateway to support fast comparison with previous readings and detect anomalies. |  |
|  |  |  | Which network device(s) should handle computing across the edge/fog layer? |  | The edge device performs real-time data processing and analytics, including: - Validate truck positioning using the vehicle detection sensor - Verify and capture valid weight readings from the load cell and weight transmitter The IoT gateway, located in the port’s central control room, performs aggregation and anomaly detection across multiple weighbridge stations before forwarding data to the cloud. |  |
|  |  |  | What data is transmitted between the IoT devices and network device(s)? |  | From sensors to the edge device: - Truck positioning status (from vehicle detection sensor) - Raw weight measurements (from digital load cell and weight transmitter) From edge device to IoT gateway: - Validated and filtered weight records and position information for further aggregation |  |
|  |  |  | What data processing and/or analysis tasks should be performed in the edge/fog layer? |  | At the edge device: - Assess if the truck is correctly positioned before capturing weight - Trigger alert upon detecting invalid or abnormal weight measurements At the IoT gateway: - Aggregate data from multiple weighbridges - Identify weight anomalies or inconsistencies across weighbridges |  |
|  |  |  | If the fog connects with a user application, what is the purpose of the connection? |  | N/A |  |
|  |  |  | If the fog connects with a system, what is the purpose of the connection? |  | Weighbridge monitoring dashboard Purpose: To provide real-time visibility of weighing operations and support operational decision-making by displaying weight data, validation status, and vehicle information. |  |
|  | Determine the connectivity to the cloud |  | Is a cloud layer required? Why? |  | Yes, the cloud layer is required to support long-term storage and historical analysis of weighbridge data collected across the port. |  |
|  |  |  | What data should be transmitted to the cloud, and is it from IoT devices or edge/fog layer devices? |  | Data transmitted from the IoT gateway to the cloud, including: - Validated and aggregated truck weight records - Weighing time and status - Vehicle licence plate numbers - Records of failed or invalid weighing attempts |  |
|  |  |  | What data processing and/or analysis tasks should be performed in the cloud? |  | - Analyse truck weighing patterns over time - Detect abnormal weight trends for specific vehicles or cargoes - Identify frequency and root causes of invalid weighing activities |  |
|  |  |  | If the cloud connects with a user application, what is the purpose of the connection? |  | Weighing History Tracker Purpose: To assist weighbridge staff in reviewing a truck’s historical weighing records for anomaly detection during the weighing process. |  |
|  |  |  | If the cloud connects with a system, what is the purpose of the connection? |  | The weighbridge analytics dashboard Purpose: To support operational analysis and fraud detection by aggregating and analysing historical weighbridge data. |  |
| IoT devices and network devices (e.g., router, hub, gateway, etc.) | Main considerations | Specific questions |  | Responses |  |
| L6A Enterprise LPR Camera (an example of a vehicle license plate recognition device) H5A Rugged PTZ Camera (an example of a security monitoring device) Dart3 Global BLE (an example of a GPS receiver) IoT gateway | Determine the connectivity between selected IoT devices, gateways, routers, etc. | Why do selected devices need to be connected with each other and to the network? |  | The GPS tracker and cameras must be connected to the IoT gateway to enable real-time tracking and security monitoring of truck movements within the port. By integrating GPS data with video feeds, the system can detect unauthorised vehicle routes, allowing for immediate security responses with visual verification. This connection ensures accurate anomaly detection, reduces manual monitoring efforts, and enhances operational security. |  |
|  | Determine communication and network technologies required to support connectivity | What are the range and coverage requirements? |  | The L6A camera supports various read distances depending on the time of day. In low-light conditions, it captures up to 250 ft (76.2 m), while in daylight, the distance extends from 20 ft to 330 ft (6.1–100.6 m). The H5A camera covers a wide surveillance area for continuous security monitoring. Dart3 Global BLE communicates with Satellites to capture a (moving) vehicle’s locations. Given the availability of network infrastructure near the port (e.g., cellular towers nearby), coverage is not a limiting factor for this GPS receiver. The network connecting the above devices should cover the entire port area, typically ranging from 20 to over 100 square kilometers, to ensure continuous and reliable data transmission. Therefore, the chosen network technology should provide stable, long-range connectivity. |  |
|  |  | What are the power consumption constraints? |  | The L6A camera consumes a maximum of 25W via Power over Ethernet (PoE). The H5A camera requires up to 71W when powered by 24V DC auxiliary supply or 90W via PoE, requiring a reliable power source to support its continuous video streaming. Dart3 Global BLE is designed for low power consumption, primarily due to its use of Bluetooth Low Energy (BLE) technology, which is optimised for minimal power usage. |  |
|  |  | What is the required data rate (how much data needs to be transmitted over time)? |  | GPS data and video feeds require high-speed transmission to support real-time monitoring and analysis. GPS data consists of small packets but must be sent at regular intervals (e.g., every few seconds) to maintain accurate vehicle tracking. In contrast, high-resolution video feeds (from the H5A camera) require a high-bandwidth network to handle large data rates without congestion, ensuring smooth, uninterrupted streaming for security monitoring. |  |
|  |  | What are the latency requirements (how fast does data need to be transmitted)? |  | Any transmission delays could impact fraud detection, route validation, and security alerts, making it crucial for GPS and video data to be delivered within milliseconds for real-time monitoring and response. Hence, low latency is essential for the port to immediately process real-time GPS data and video feeds to make timely decisions on vehicle movements. |  |
|  |  | What are the security and reliability requirements? |  | The GPS receiver and cameras handle sensitive access data, including truck licence plate numbers, vehicle locations, and silo locations. To prevent unauthorised access or tampering, the network must ensure strong encryption, secure authentication, and controlled access to transmitted data. For reliability, the network must remain operational in harsh port environments, where strong winds, heavy rain, and high humidity could impact connectivity. |  |
|  |  | What are the security and reliability requirements? |  | <optional> |  |
|  |  | What are the security and reliability requirements? |  | LTE-M is chosen to connect Dart3 Global BLE to the IoT Gateway for its support to fast data transmission, low bandwidth, and wide coverage. Since the GPS receiver constantly calculates vehicle location, the network must support frequent updates with minimal delay (i.e., low latency). With cellular towers available near the port, LTE-M ensures reliable and uninterrupted connectivity for real-time vehicle tracking across the entire port. Ethernet for connecting two types of cameras with the IoT Gateway - Ethernet is selected due to its high speed, low latency, and stable connectivity, essential for transmitting high-resolution video feeds. Video data requires high bandwidth to prevent congestion, and Ethernet ensures smooth, interference-free transmission, which is critical for real-time security monitoring. - With Power over Ethernet (PoE), cameras receive both power and data through a single cable, simplifying installation and reducing infrastructure costs. Additionally, Ethernet provides strong encryption and network security features, protecting sensitive video feeds from cyber threats. Unlike wireless alternatives, Ethernet maintains consistent performance and is unaffected by environmental interference from metal structures or weather conditions, ensuring reliable and continuous monitoring. |  |
|  | Determine the connectivity between IoT devices and network devices in the edge/fog layer | Is an edge/fog layer required? Why? |  | Yes, a fog layer is required to enable real-time validation of a truck’s location and route within the port using GPS data and license plate images. This supports low-latency detection of route deviations and suspicious vehicle movements. |  |
|  |  | Which network device(s) should handle computing across the edge/fog layer? |  | The IoT gateway located at the central control centre performs fog computing tasks, including: - Receiving live GPS data and LPR camera images - Matching vehicle positions with predefined route - Detecting route deviations or suspicious vehicle movements in real time |  |
|  |  | What data is transmitted between the IoT devices and network device(s)? |  | From L6A LPR Camera to IoT gateway: - vehicle license plate images, captured timestamps, and camera ID From Dart3 GPS Receiver to IoT gateway: - GPS coordinates and associated vehicle identifiers (e.g., license plate number) at regular intervals |  |
|  |  | What data processing and/or analysis tasks should be performed in the edge/fog layer? |  | - Match live GPS coordinates with vehicle routes - Associate GPS data with the license plate captured by the LPR camera - Detect unauthorised or unexpected vehicle movements based on vehicle routes (GPS and vehicle photos) |  |
|  |  | If the fog connects with a user application, what is the purpose of the connection? |  | N/A |  |
|  |  | If the fog connects with a system, what is the purpose of the connection? |  | Vehicle tracking system Purpose: To provide real-time vehicle location tracking and generate alerts for route deviations or suspicious movements. |  |
|  | Determine the connectivity to the cloud | Is a cloud layer required? Why? |  | Yes, the cloud layer is required for long-term storage, route history analysis, and fleet movement reporting across time and port-wide locations. |  |
|  |  | What data should be transmitted to the cloud, and is it from IoT devices or edge/fog layer devices? |  | Data transmitted from the IoT gateway includes: - Aggregated GPS location history per vehicle - Detected route anomalies and alert records |  |
|  |  | What data processing and/or analysis tasks should be performed in the cloud? |  | - Analyse vehicle route patterns and dwell times within the port - Detect suspicious vehicle behavior or repeated violations of routes - Generate historical reports for operational efficiency and security auditing |  |
|  |  | If the cloud connects with a user application, what is the purpose of the connection? |  | Security alert application Purpose: To notify port security staff when a vehicle significantly deviates from its pre-defined route. |  |
|  |  | If the cloud connects with a system, what is the purpose of the connection? |  | Vehicle monitoring dashboard Purpose: To support route monitoring and investigation by analysing historical vehicle movement data. |  |

Resources:
https://www.nokia.com/networks/use-cases/private-lte-based-networks-for-terminal-operators/

https://www.dusuniot.com/blog/router-vs-gateway-which-to-choose-for-my-iot-project/

https://www.fs.com/blog/similarities-and-differences-between-router-and-gateway-1303.html

https://www.pusr.com/blog/Differences-between-IoT-gateway-devices-and-IoT-cellular-modem

https://freeeway.com/what-is-lte-m-and-how-does-it-differ-from-traditional-lte/